Kubernetes Cluster Setup

kubeadm on AWS EC2 t3.medium

Complete Step-by-Step Reference Guide

| Kubernetes Version | v1.33.9 |
| --- | --- |
| CNI Plugin | Calico v3.30.1 |
| Container Runtime | containerd |
| Instance Type | AWS EC2 t3.medium (4x) |
| OS | Ubuntu 22.04 LTS |

# 1. Cluster Architecture

This setup uses 1 Control Plane node and 3 Worker nodes — all on AWS EC2 t3.medium instances (2 vCPU, 4 GB RAM each).

| Node Name | Role | Instance Type | Specs |
| --- | --- | --- | --- |
| k8s-master (ip-172-31-2-225) | Control Plane | t3.medium | 2 vCPU / 4 GB RAM |
| k8s-worker-1 (ip-172-31-0-197) | Worker Node | t3.medium | 2 vCPU / 4 GB RAM |
| k8s-worker-2 (ip-172-31-1-224) | Worker Node | t3.medium | 2 vCPU / 4 GB RAM |
| k8s-worker-3 (ip-172-31-2-139) | Worker Node | t3.medium | 2 vCPU / 4 GB RAM |

NOTE: All kubectl commands must be run on the Control Plane node only, unless stated otherwise.

# 2. EC2 Security Group Configuration

Before installing anything, configure the Security Group inbound rules in AWS Console. These ports allow Kubernetes components to communicate with each other.

### Control Plane Node Ports

| Port / Range | Protocol | Purpose | What it does in background |
| --- | --- | --- | --- |
| 6443 | TCP | Kubernetes API Server | All kubectl commands and worker nodes talk to master through this port |
| 2379-2380 | TCP | etcd Server | etcd is the key-value store that holds all cluster state and config |
| 10250 | TCP | Kubelet API | API that lets master communicate with the kubelet agent on each node |
| 10257 | TCP | kube-controller-manager | Controller that watches cluster state and reconciles desired vs actual |
| 10259 | TCP | kube-scheduler | Scheduler that assigns pods to worker nodes based on resource availability |
| 30000-32767 | TCP | NodePort Services | External traffic enters the cluster through these ports via NodePort services |
| 22 | TCP | SSH | Remote terminal access to manage the instance |

### Worker Node Ports

| Port / Range | Protocol | Purpose | What it does in background |
| --- | --- | --- | --- |
| 10250 | TCP | Kubelet API | Master sends pod scheduling and health check instructions to kubelet here |
| 30000-32767 | TCP | NodePort Services | External users can reach services on any worker node via these ports |
| 22 | TCP | SSH | Remote terminal access |

TIP: Also add an inbound rule: All Traffic from source = the Security Group itself. This allows all nodes in the group to talk freely with each other.

# 3. Common Setup — Run on ALL 4 Nodes

SSH into each node individually and run all commands in this section. These steps prepare the OS for Kubernetes before installation.

## 3.1 Disable Swap

WHERE TO RUN: All 4 nodes (master + 3 workers)

WHY: Kubernetes requires swap to be disabled. If swap is enabled, the kubelet will refuse to start. Swap causes unpredictable memory behavior which breaks Kubernetes scheduling.

sudo swapoff -a

sudo sed -i '/ swap / s/^\(.*\)$/#\1/g' /etc/fstab

swapoff -a disables swap immediately. The sed command comments out the swap entry in /etc/fstab so swap stays off after a reboot.

## 3.2 Load Kernel Modules

WHERE TO RUN: All 4 nodes

WHY: overlay enables the overlay filesystem used by containers. br_netfilter allows iptables to see bridged network traffic, which is required for pod-to-pod networking.

cat <<EOF | sudo tee /etc/modules-load.d/k8s.conf

overlay

br_netfilter

EOF

sudo modprobe overlay

sudo modprobe br_netfilter

## 3.3 Configure Sysctl (Network Settings)

WHERE TO RUN: All 4 nodes

WHY: These kernel parameters ensure that network bridge traffic passes through iptables, which is required for Kubernetes networking and service routing to work correctly.

cat <<EOF | sudo tee /etc/sysctl.d/k8s.conf

net.bridge.bridge-nf-call-iptables  = 1

net.bridge.bridge-nf-call-ip6tables = 1

net.ipv4.ip_forward                 = 1

EOF

sudo sysctl --system

Verify ip_forward is enabled (should output 1):

cat /proc/sys/net/ipv4/ip_forward

# Expected output: 1

If it outputs 0, run this immediate fix before proceeding:

sudo sysctl -w net.ipv4.ip_forward=1

## 3.4 Install containerd (Container Runtime)

WHERE TO RUN: All 4 nodes

WHY: containerd is the container runtime interface (CRI) that Kubernetes uses to run containers. Without a CRI, Kubernetes cannot start or manage any pods.

sudo apt-get update

sudo apt-get install -y ca-certificates curl gnupg

sudo install -m 0755 -d /etc/apt/keyrings

curl -fsSL https://download.docker.com/linux/ubuntu/gpg | \

sudo gpg --dearmor -o /etc/apt/keyrings/docker.gpg

sudo chmod a+r /etc/apt/keyrings/docker.gpg

echo "deb [arch=$(dpkg --print-architecture) signed-by=/etc/apt/keyrings/docker.gpg] https://download.docker.com/linux/ubuntu $(. /etc/os-release && echo \"$VERSION_CODENAME\") stable" | sudo tee /etc/apt/sources.list.d/docker.list > /dev/null

sudo apt-get update

sudo apt-get install -y containerd.io

## 3.5 Configure containerd with SystemdCgroup

WHERE TO RUN: All 4 nodes

WHY: By default, containerd uses cgroupfs for cgroup management. Kubernetes recommends SystemdCgroup=true so that both kubelet and containerd use the same cgroup driver (systemd), preventing resource conflicts.

sudo mkdir -p /etc/containerd

containerd config default | sudo tee /etc/containerd/config.toml

sudo sed -i 's/SystemdCgroup = false/SystemdCgroup = true/' /etc/containerd/config.toml

sudo systemctl restart containerd

sudo systemctl enable containerd

## 3.6 Install kubeadm, kubelet, kubectl

WHERE TO RUN: All 4 nodes

| Tool | What it does |
| --- | --- |
| kubeadm | Bootstraps the cluster — initializes master and joins workers |
| kubelet | Agent that runs on every node — starts/stops containers as instructed by master |
| kubectl | CLI tool to interact with the cluster API server (mainly used on master) |

sudo apt-get update

sudo apt-get install -y apt-transport-https ca-certificates curl gpg

curl -fsSL https://pkgs.k8s.io/core:/stable:/v1.33/deb/Release.key | \

sudo gpg --dearmor -o /etc/apt/keyrings/kubernetes-apt-keyring.gpg

echo "deb [signed-by=/etc/apt/keyrings/kubernetes-apt-keyring.gpg] https://pkgs.k8s.io/core:/stable:/v1.33/deb/ /" | sudo tee /etc/apt/sources.list.d/kubernetes.list

sudo apt-get update

sudo apt-get install -y kubelet kubeadm kubectl

sudo apt-mark hold kubelet kubeadm kubectl

sudo systemctl enable --now kubelet

NOTE: apt-mark hold prevents accidental upgrades of these packages, which could break cluster compatibility between nodes.

# 4. Initialize Control Plane — Master Node ONLY

WHERE TO RUN: Master node only (ip-172-31-2-225)

WHY: kubeadm init sets up all the control plane components: API server, etcd, controller-manager, and scheduler. It generates TLS certificates, kubeconfig files, and creates the initial cluster configuration.

MASTER_IP=$(hostname -I | awk '{print $1}')

sudo kubeadm init \

--apiserver-advertise-address=$MASTER_IP \

--pod-network-cidr=192.168.0.0/16 \

--cri-socket /run/containerd/containerd.sock

| Flag | What it does |
| --- | --- |
| --apiserver-advertise-address | IP that other nodes use to reach the API server. Use private IP of master. |
| --pod-network-cidr=192.168.0.0/16 | IP range assigned to pods. Must match what Calico CNI expects (192.168.0.0/16). |
| --cri-socket | Tells kubeadm which container runtime to use. Points to containerd socket. |

## 4.1 Set up kubectl Access

WHERE TO RUN: Master node only — run immediately after kubeadm init

WHY: kubectl needs a kubeconfig file to know which cluster to connect to and how to authenticate. Without this step, kubectl will try localhost:8080 and fail with connection refused.

mkdir -p $HOME/.kube

sudo cp -i /etc/kubernetes/admin.conf $HOME/.kube/config

sudo chown $(id -u):$(id -g) $HOME/.kube/config

# 5. Install Calico CNI — Master Node ONLY

WHERE TO RUN: Master node only

WHY: Without a CNI plugin, pods cannot communicate with each other across nodes. Calico provides pod networking by creating a virtual network overlay. Without it, nodes will stay in NotReady state.

kubectl create -f https://raw.githubusercontent.com/projectcalico/calico/v3.30.1/manifests/operator-crds.yaml

kubectl create -f https://raw.githubusercontent.com/projectcalico/calico/v3.30.1/manifests/tigera-operator.yaml

kubectl create -f https://raw.githubusercontent.com/projectcalico/calico/v3.30.1/manifests/custom-resources.yaml

| Manifest | What it installs |
| --- | --- |
| operator-crds.yaml | Custom Resource Definitions — extends Kubernetes API to understand Calico objects |
| tigera-operator.yaml | The Tigera operator that manages the Calico lifecycle |
| custom-resources.yaml | The actual Calico network configuration using pod CIDR 192.168.0.0/16 |

Wait for all Calico pods to be Running before joining worker nodes:

watch kubectl get pods -n calico-system

# 6. Join Worker Nodes — Each Worker Node

WHERE TO RUN: Each of the 3 worker nodes individually

WHY: The join command registers the worker with the master. It uses a token (valid 24 hrs) and a certificate hash to authenticate. Once joined, kubelet on the worker connects to the API server and receives pod scheduling instructions.

## 6.1 Get the Join Command (on Master)

If your token expired (tokens expire in 24 hours), regenerate:

kubeadm token create --print-join-command

## 6.2 Run on Each Worker Node

IMPORTANT: Always use sudo. Running without sudo gives: ERROR IsPrivilegedUser: user is not running as root.

sudo kubeadm join <MASTER_PRIVATE_IP>:6443 \

--token <token> \

--discovery-token-ca-cert-hash sha256:<hash> \

--cri-socket /run/containerd/containerd.sock

## 6.3 If Worker Already Has Old Config (Reset First)

If the worker was joined before or had a failed attempt, reset it first:

sudo kubeadm reset -f

sudo rm -rf /etc/kubernetes /var/lib/kubelet /var/lib/cni /etc/cni

sudo systemctl restart containerd kubelet

Then run the join command again.

# 7. Verify the Cluster — Master Node

WHERE TO RUN: Master node only

kubectl get nodes -o wide

Expected output — all nodes should show Ready:

NAME              STATUS   ROLES           AGE   VERSION

ip-172-31-2-225   Ready    control-plane   10m   v1.33.9

ip-172-31-0-197   Ready    <none>          5m    v1.33.9

ip-172-31-1-224   Ready    <none>          5m    v1.33.9

ip-172-31-2-139   Ready    <none>          5m    v1.33.9

kubectl get pods -A                    # All pods across all namespaces

kubectl get pods -n calico-system      # Check Calico health

kubectl cluster-info                   # API server and CoreDNS info

# 8. Quick Command Reference

| Command | Run On | What it does |
| --- | --- | --- |
| kubectl get nodes | Master | Lists all nodes and their status |
| kubectl get pods -A | Master | Lists all pods across all namespaces |
| kubectl get pods -o wide | Master | Shows pods with IP and node assignment |
| kubectl describe node <name> | Master | Detailed info about a node including events |
| kubectl logs <pod-name> | Master | View logs of a running pod |
| kubectl get svc | Master | List all services and their exposed ports |
| kubeadm token create --print-join-command | Master | Generate a new worker join command |
| sudo kubeadm reset -f | Any node | Completely reset kubeadm config on that node |
| sudo systemctl status kubelet | Any node | Check if kubelet is running on the node |
| sudo systemctl restart containerd | Any node | Restart the container runtime |

# 9. Troubleshooting

| Error | Cause | Fix |
| --- | --- | --- |
| connection refused localhost:8080 | kubeconfig not set up | Run: mkdir -p $HOME/.kube && sudo cp /etc/kubernetes/admin.conf $HOME/.kube/config |
| ERROR IsPrivilegedUser | Forgot sudo on join | Run the join command with sudo |
| FileAvailable--etc-kubernetes-kubelet.conf | Node already joined before | Run kubeadm reset -f then rejoin |
| Port 10250 is in use | Old kubelet still running | Run kubeadm reset -f then rejoin |
| Node stays NotReady | CNI not installed or failing | Check: kubectl get pods -n calico-system |
| Malformed entry in sources.list | Line break in echo command | Delete file and re-add repo URL as single line |
| Token expired | Tokens expire after 24 hours | Run: kubeadm token create --print-join-command on master |
| ERROR FileContent ip_forward not set to 1 | Step 3.3 sysctl was skipped or did not apply | Run: sudo sysctl -w net.ipv4.ip_forward=1 then re-run Step 3.3 fully |

End of Document